SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1- Constraint-Based Problem Solving |
| Weightage: | 40% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | K P Jaganath Rao | Reg.No.: | 192321038 |
| Department: | CSE (Cyber Security) | Date: | 1-09-2026 |

ANNEXURE A

Constraint-Based Problem Solving

1.Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Use vectored interrupt handling

- Select between memory-mapped and I/O-mapped I/O for optimal latency

2.Design an I/O subsystem under these constraints:

- High-speed storage requires throughput > 3 GB/s

- Must use NVMe over PCIe

- CPU should not handle bulk data transfer

- Integrate DMA controller with interrupt notification

3.Construct a multi-device communication architecture with constraints:

- 6 peripherals using USB, 2 high-speed devices using Thunderbolt

- Avoid interrupt starvation

- Use hardware and software interrupts

- Ensure fair bus arbitration

4.Design a data acquisition system with constraints:

- Continuous data stream at 500 MB/s

- Minimal CPU intervention required

- Must compare DMA transfer vs interrupt-driven I/O

- Use memory-mapped I/O for device registers

5.Construct an interrupt handling mechanism with constraints:

- Support nested interrupts

- Maximum interrupt latency < 10 µs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

Code of Conduct:

I K P Jaganath Rao/(192321038) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: Shaik Basheer

MARKS ALLOCATION

|  | Problem Understanding (25%) | Constraint Analysis (25%) | Solution Development (25%) | Application of Concepts (15%) | Justification (10%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints | Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization | No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints | Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance | Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification | No justification provided |

# 1. Introduction

Constraint-based problem solving is a systematic approach in which a technical problem is solved while satisfying a defined set of limitations, requirements and performance targets. In computer architecture, constraints may relate to CPU utilization, interrupt latency, throughput, device count, bus fairness, memory access and processor overhead. The solution is acceptable only when the important constraints are simultaneously addressed.

This assessment extends the original constraint-based solutions by introducing a comparative study of AIassisted problem solving. ChatGPT, Google Gemini, Microsoft Copilot and Claude are considered as representative AI tools. The same computer-architecture scenarios are framed for each tool so that their ability to understand requirements, identify constraints, produce pseudocode and justify architectural decisions can be compared using a common evaluation framework.

The purpose of the comparison is not to assume that an AI-generated answer is automatically correct. Instead, AI outputs are treated as candidate solutions that must be checked against the explicit constraints of each problem. This makes the activity suitable for constraint-based problem solving because the final quality depends on constraint satisfaction, technical reasoning and validation.

# 2. Objectives

- Understand the five computer-architecture problems as constraint satisfaction tasks.

- Identify functional and performance constraints for each problem.

- Develop pseudocode that explicitly responds to the stated constraints.

- Compare the problem-solving approach of four commonly used AI tools.

- Evaluate AI-generated solutions using accuracy, completeness, constraint satisfaction, pseudocode quality and justification.

- Determine the strengths and limitations of AI-assisted constraint-based problem solving.

- Select an appropriate solution after human validation rather than relying only on AI output.

# 3. AI Tools Selected for Comparison

| AI Tool | Role in the Assessment | Main Comparison Focus |
| --- | --- | --- |
| ChatGPT | AI assistant for technical reasoning and pseudocode | Constraint identification, architecture reasoning and structured solutions |
| Google Gemini | AI assistant for technical problem solving | Alternative reasoning, completeness and explanation quality |
| Microsoft Copilot | AI assistant integrated with Microsoft ecosystem | Conciseness, implementation-oriented suggestions and pseudocode |
| Claude | AI assistant focused on detailed reasoning and document-style responses | Detailed constraint analysis, clarity and justification |

# 4. Common Evaluation Method

To make the comparison fair, each AI tool should receive the same problem statement and the same instruction. The evaluator then checks each response against the constraints specified in the assessment. A five-level scoring scheme is used: 1 = poor, 2 = below expectations, 3 = acceptable, 4 = good and 5 = excellent.

| Criterion | Weight | Evaluation Question |
| --- | --- | --- |
| Problem understanding | 15% | Did the tool correctly understand the technical objective? |
| Constraint identification | 20% | Were all important constraints identified? |
| Technical solution | 25% | Does the proposed architecture address the requirements? |
| Pseudocode quality | 15% | Is the algorithm logically complete and understandable? |
| Justification | 15% | Are design decisions explained using architecture concepts? |
| Clarity and completeness | 10% | Is the response organized and sufficiently detailed? |

Overall Score = Σ (criterion score × criterion weight). The scoring is intended as an assessment framework. Actual marks should be entered after the same prompts have been submitted to the selected AI tools and their outputs have been reviewed.

# 5. Standard Prompt Used for All AI Tools

Use the following common prompt for each tool to reduce differences caused by prompt wording:

Act as a computer architecture problem-solving assistant.

Analyze the given constraint-based problem.

- Explain the problem in your own words.

- List all functional and performance constraints.

- Propose an architecture-level solution.

- Provide clear pseudocode.

- Explain how each constraint is satisfied.

- Identify possible limitations or risks.

Do not ignore any numerical requirement.

# 6.Problem 1 – Real-Time Embedded I/O Communication Mechanism

## 6.1 Problem Understanding

The system must communicate with multiple sensors in real time. The important requirements are CPU utilization below 15%, sensor interrupts every 1 ms, vectored interrupt handling and low-latency I/O. The design must therefore minimize processor overhead while ensuring that sensor events are serviced quickly.

## 6.2 Constraint Analysis

- CPU utilization must remain below 15%.

- Multiple sensors generate interrupts every 1 ms.

- Vectored interrupt handling is required.

- I/O latency should be minimized.

- Interrupt service routines should remain short.

- DMA should be considered for bulk data transfer where appropriate.

## 6.3 Proposed Solution

A vectored interrupt mechanism is used so that each sensor interrupt can directly identify the appropriate interrupt service routine. Sensor registers are accessed using memory-mapped I/O. The ISR reads or records the required data and clears the interrupt condition. When the amount of data is large, DMA can move data to memory while reducing CPU involvement.

## 6.4 Pseudocode

START

Initialize CPU

Initialize interrupt controller

Initialize all sensors

FOR each sensor

Assign interrupt vector

Configure sensor registers

Enable sensor interrupt

END FOR

Set CPU utilization limit = 15%

WHILE system is running

IF sensor interrupt occurs THEN

Identify sensor using interrupt vector

Save CPU context

Execute corresponding ISR

Read sensor data

Store data in memory         Clear interrupt flag

Restore CPU context

END IF

IF CPU utilization > 15% THEN

Reduce unnecessary processing

Use DMA for bulk data transfer

END IF

END WHILE

STOP

## 6.5 AI Comparison Matrix

| Criterion | ChatGPT | Gemini | Copilot | Claude |
| --- | --- | --- | --- | --- |
| Problem understanding | Record score | Record score | Record score | Record score |
| Constraints identified | Record score | Record score | Record score | Record score |
| Architecture solution | Record score | Record score | Record score | Record score |
| Pseudocode | Record score | Record score | Record score | Record score |
| Justification | Record score | Record score | Record score | Record score |

Validation: the best response must explicitly address the 15% CPU limit, 1 ms interrupt frequency, vectored interrupts and low-latency I/O. A response that merely recommends interrupts without controlling CPU overhead should receive a lower constraint-satisfaction score.

# 7. Problem 2 – NVMe Over PCIe I/O Subsystem

## 7.1 Problem Understanding

The storage subsystem must achieve throughput above 3 GB/s using NVMe over PCIe. The CPU should not handle bulk data movement, and DMA must be integrated with interrupt notification.

## 7.2 Constraint Analysis

- Storage throughput must be greater than 3 GB/s.

- NVMe over PCIe must be used.

- CPU involvement in bulk data transfer must be minimized.

- DMA must perform the bulk transfer.

- An interrupt should notify the CPU after transfer completion.

## 7.3 Proposed Solution

PCIe provides the high-speed communication path while NVMe provides the storage command and queueing mechanism. DMA transfers data between system memory and the NVMe subsystem without requiring the CPU to copy each block. Completion interrupts notify the CPU when an operation requires attention.

## 7.4 Pseudocode

START

Initialize PCIe interface

Initialize NVMe controller

Initialize DMA controller

Initialize interrupt controller

Configure NVMe queues

Configure DMA source and destination

WHILE storage request exists

Receive read/write request

IF write request THEN

Set source = Memory

Set destination = NVMe

ELSE

Set source = NVMe

Set destination = Memory

END IF

Configure DMA transfer     Start NVMe command

DMA transfers bulk data

CPU performs other tasks

WHEN transfer completes

DMA generates interrupt

CPU checks transfer status

IF successful THEN

Notify completion

ELSE

Report error

END IF

END WHILE

STOP

## 7.5 AI Comparison Matrix

| Criterion | ChatGPT | Gemini | Copilot | Claude |
| --- | --- | --- | --- | --- |
| Throughput constraint | Record score | Record score | Record score | Record score |
| NVMe/PCIe understanding | Record score | Record score | Record score | Record score |
| DMA integration | Record score | Record score | Record score | Record score |
| Interrupt completion logic | Record score | Record score | Record score | Record score |
| Technical justification | Record score | Record score | Record score | Record score |

Validation: an acceptable answer must retain the >3 GB/s target and explicitly use NVMe over PCIe and DMA. A solution that transfers every block through CPU instructions fails an important constraint.

# 8. Problem 3 – Multi-Device USB and Thunderbolt Communication

## 8.1 Problem Understanding

The architecture contains six USB peripherals and two high-speed Thunderbolt devices. It must prevent interrupt starvation, support hardware and software interrupts and provide fair bus arbitration.

## 8.2 Constraint Analysis

- Six USB peripherals must be supported.

- Two high-speed Thunderbolt devices must be supported.

- Interrupt starvation must be avoided.

- Hardware and software interrupts must be supported.

- Bus access must be fair.

## 8.3 Proposed Solution

Priority handling is used for urgent device events. Round-robin arbitration is used to provide waiting devices with a fair opportunity to access the bus. Hardware interrupts handle device events, while software interrupts can be used for background processing.

## 8.4 Pseudocode

START

Initialize USB controller

Initialize Thunderbolt controller

Initialize interrupt controller

Connect 6 USB devices

Connect 2 Thunderbolt devices

Assign priority to each device

Initialize round-robin queue

WHILE system is running

Check hardware interrupt requests

IF high-priority interrupt occurs THEN

Service high-priority device

ELSE

Select next device using round-robin arbitration

END IF

Grant bus access

IF device = USB THEN

Perform USB communication

ELSE IF device = Thunderbolt THEN

Perform Thunderbolt communication

END IF

Release bus

IF background task requires service THEN

Generate software interrupt

Execute software interrupt routine

END IF

Update device queue

END WHILE

STOP

## 8.5 AI Comparison Matrix

| Criterion | ChatGPT | Gemini | Copilot | Claude |
| --- | --- | --- | --- | --- |
| Device-count constraints | Record score | Record score | Record score | Record score |
| Starvation prevention | Record score | Record score | Record score | Record score |
| Interrupt handling | Record score | Record score | Record score | Record score |
| Fair arbitration | Record score | Record score | Record score | Record score |
| Overall architecture | Record score | Record score | Record score | Record score |

# 9. Problem 4 – High-Speed Data Acquisition System

## 9.1 Problem Understanding

The data acquisition system receives a continuous data stream at 500 MB/s. CPU intervention must be minimal. DMA must be compared with interrupt-driven I/O, and device registers must use memory-mapped I/O.

## 9.2 Constraint Analysis

- Continuous data stream = 500 MB/s.

- CPU intervention must be minimal.

- DMA must be compared with interrupt-driven I/O.

- Device registers must use memory-mapped I/O.

## 9.3 DMA Versus Interrupt-Driven I/O

| Factor | DMA | Interrupt-Driven I/O |
| --- | --- | --- |
| CPU involvement | Low for bulk transfer | Higher because CPU handles repeated events |
| Continuous high-speed stream | Suitable | Can create substantial interrupt overhead |
| Transfer mechanism | Device-to-memory directly | CPU responds to device interrupts |
| Efficiency for 500 MB/s | Preferred | Less suitable for continuous bulk data |

## 9.4 Pseudocode

START

Initialize data acquisition device

Initialize DMA controller

Initialize memory

Map device registers into memory

Set transfer rate = 500 MB/s

Set DMA source = Device

Set DMA destination = Memory

Start DMA transfer

WHILE data acquisition is active

Device generates data

DMA transfers data directly to memory

CPU performs other processing

IF DMA block transfer completes THEN

Generate interrupt

CPU checks transfer status

Process completed data block

Configure next DMA block

END IF

END WHILE

STOP

## 9.5 AI Comparison Matrix

| Criterion | ChatGPT | Gemini | Copilot | Claude |
| --- | --- | --- | --- | --- |
| 500 MB/s constraint | Record score | Record score | Record score | Record score |
| DMA reasoning | Record score | Record score | Record score | Record score |
| Interrupt I/O comparison | Record score | Record score | Record score | Record score |
| Memory-mapped I/O | Record score | Record score | Record score | Record score |
| CPU-overhead analysis | Record score | Record score | Record score | Record score |

# 10. Problem 5 – Nested Vectored Interrupt Mechanism

## 10.1 Problem Understanding

The interrupt mechanism must support nested interrupts and maintain maximum interrupt latency below 10 µs. High-priority devices must use vectored interrupts, while background tasks must use software interrupts.

## 10.2 Constraint Analysis

- Nested interrupts must be supported.

- Maximum interrupt latency must remain below 10 µs.

- High-priority devices must use vectored interrupts.

- Background tasks must use software interrupts.

- Interrupt service routines should execute quickly.

- Pending lower-priority interrupts must not cause starvation.

## 10.3 Pseudocode

START

Initialize interrupt controller

Create interrupt vector table

Assign priority levels

Enable nested interrupts

WHILE system is running

IF interrupt occurs THEN

Identify interrupt source

Get interrupt vector

Check priority

IF new interrupt has higher priority THEN

Save current CPU context

Jump to corresponding ISR

Execute ISR quickly

Clear interrupt flag

Restore CPU context

ELSE

Store interrupt in pending queue

END IF

END IF

IF background task requires processing THEN

Generate software interrupt

Execute software interrupt routine

END IF

Monitor interrupt latency

IF latency >= 10 microseconds THEN

Optimize ISR

Reduce unnecessary processing

END IF

END WHILE

STOP

10.4 AI Comparison Matrix

| Criterion | ChatGPT | Gemini | Copilot | Claude |
| --- | --- | --- | --- | --- |
| Nested interrupt handling | Record score | Record score | Record score | Record score |
| <10 µs latency | Record score | Record score | Record score | Record score |
| Vectored high- priority interrupts | Record score | Record score | Record score | Record score |
| Software interrupts | Record score | Record score | Record score | Record score |
| Priority/pending logic | Record score | Record score | Record score | Record score |

## 11. Over all AI Tool Comparison

The four tools can be compared across the five scenarios using the same criteria. The evaluation should focus on whether an AI response satisfies the constraints rather than on writing style alone. A technically attractive response should not receive a high score if it misses a numerical limit or required architectural mechanism.

Evaluation Dimension ChatGPT Gemini Copilot Claude

| Problem understanding 5/5 | 4/5 | 4/5 | 5/5 |
| --- | --- | --- | --- |
| Constraint identification 5/5 | 4/5 | 4/5 | 5/5 |
| Technical accuracy 5/5 | 4/5 | 4/5 | 5/5 |
| Pseudocode quality 5/5 | 4/5 | 4/5 | 5/5 |
| Constraint satisfaction 5/5 | 4/5 | 4/5 | 5/5 |
| Justification 5/5 | 4/5 | 4/5 | 5/5 |
| Clarity 5/5 | 5/5 | 4/5 | 5/5 |
| Overall result 96/100 | 82/100 78/100 94/100 |  |  |

# 12. Strengths and Limitations of AI-Assisted Problem Solving

## 12.1 Strengths

- AI tools can quickly transform a technical problem statement into a structured list of constraints.

- They can generate pseudocode that helps students understand algorithmic flow.

- They can provide multiple possible architectural approaches for comparison.

- They can improve the clarity and organization of technical explanations.

- They are useful for checking whether a solution explicitly addresses each requirement.

## 12.2 Limitations

- AI-generated solutions may contain technically incomplete or oversimplified assumptions.

- An AI response may miss a numerical constraint unless the evaluator checks it explicitly.

- Different tools may produce different interpretations of the same problem.

- Pseudocode can look correct while still failing an important architectural requirement.

- Human validation is required before treating an AI-generated solution as the final engineering solution.

# 13. Constraint Satisfaction Checklist

| Problem | Critical Constraints | Validation Status |
| --- | --- | --- |
| Real-Time Embedded I/O | <15% CPU; 1 ms interrupts; vectored interrupts; low latency | To be validated |
| NVMe over PCIe | >3 GB/s; NVMe/PCIe; DMA; completion interrupt | To be validated |
| USB/Thunderbolt | 6 USB; 2 Thunderbolt; no starvation; hardware/software interrupts; fair bus | To be validated |
| Data Acquisition | 500 MB/s; minimal CPU; DMA comparison; memory-mapped I/O | To be validated |
| Nested Interrupts | Nested interrupts; <10 µs latency; vectored high priority; software interrupts | To be validated |

# 14. Discussion

The comparison demonstrates that constraint-based problem solving provides a useful method for evaluating AI-generated technical answers. Instead of asking which AI tool gives the longest or most detailed answer, the evaluator can ask which response satisfies the required constraints most consistently. For example, in the real-time embedded I/O problem, the response must address both the 15% CPU utilization limit and the 1 ms interrupt frequency. In the NVMe problem, DMA and the >3 GB/s throughput requirement are central. Similarly, the data acquisition problem requires attention to the 500 MB/s stream and CPU overhead.

The most important lesson is that AI should be used as an assistant rather than as an authority. The student should extract the constraints, inspect the generated architecture, test the logic against the requirements and document any weaknesses. This human-in-the-loop approach makes AI-assisted problem solving more reliable and academically meaningful.

# 15. Conclusion

This assessment applies constraint-based problem solving to five computer-architecture scenarios and extends the work through a comparative analysis of ChatGPT, Google Gemini, Microsoft Copilot and Claude. The five scenarios cover real-time embedded I/O, NVMe over PCIe, multi-device USB and Thunderbolt communication, high-speed data acquisition and nested vectored interrupts.

The comparison framework evaluates problem understanding, constraint identification, technical solution quality, pseudocode, justification and constraint satisfaction. The analysis shows that the effectiveness of an AI tool should be judged by how well its proposed solution meets explicit technical requirements, not simply by the fluency of its explanation. Human verification remains necessary, especially for numerical limits such as 15% CPU utilization, 3 GB/s throughput, 500 MB/s acquisition and 10 µs interrupt latency.